# Attendance System User Guide

This guide explains how to use the RFID-based Attendance System integrated with a web dashboard.

1. System Overview

This system uses ESP32, RFID reader (MFRC522), OLED display, and buzzer to read card IDs and send attendance data to a PHP-MySQL web server. Each device is linked to a location via MAC address.

- Device scanner: Connect to Wi-Fi AP and scan attendance using MiFare RFID card.

- MiFare RFID card: Generic and low-cost card

2. Setup Instructions

2.1 Device Setup (ESP32)

- Only if no WiFi AP configured or unable to connect to the network or move to a new network ESP32 will create its own access point called AttendanceAP.

- Connect to AttendanceAP, open web browser and navigate to http://192.168.4.1 to enter your Wi-Fi credentials (2.4 GHz network only) and save. On mobile device please disconnect your data option and only connect to the AP.

- ESP32 will reboot and try to connect to the internet.

- If connected successfully, the OLED will display the device MAC and prompt for card scanning.

- Move closer to the WiFi AP to get better connectivity if unable to connect.

2.2 Register Device

- If no Device scanner available yet in the dashboard, please add one.

- Log in to the web dashboard. (https://slumberjer.com/ipgktar) username: admin, password: abc123

- Go to Register Device (first time only for new device only) using device MAC address. The address is on the screen with all capital letters and numbers.

- Enter device MAC address (without colons) and assign a location.

2.3 Register Users

- Go to Registered Users (first time only for new user).

- Click Register New User.

- Enter card ID (shown when scanning on unregistered card), user name, and optional student ID.

3. Recording Attendance

- When a registered card is scanned on a registered device:

  - Check-in is recorded if no entry yet today.

  - Check-out is recorded if check-in already exists.

- If the card is not registered, a message will prompt to register and use the admin dashboard to register the card.

4. Viewing Attendance

- Go to Dashboard (https://slumberjer.com/ipgktar/dashboard.php).

- Select a device and date.

- View attendance with card ID, username, time-in, and time-out.

- You can delete individual entries.

- Page auto-refreshes every 5 seconds if a device is selected.

5. Admin Notes

- Timezone is set to Asia/Kuala_Lumpur.

- Data is stored in:

  - tbl_user for user information

  - tbl_attendance for attendance logs

  - tbl_devices for registered devices

6. Troubleshooting

- No Wi-Fi: ESP32 will fall back to AP mode.

- MAC Not Found: Register the device first.

- Card Not Found: Register user card in the user panel.

For more info or customization, contact the system administrator.

Wiring Diagram

Here's how to connect each component to the ESP32:

| MFRC522 | ESP32 |  |
| --- | --- | --- |
| SDA | D21 |  |
| SCK | D18 |  |
| MOSI | D23 |  |
| MISO | D19 |  |
| IRQ | Not connected |  |
| GND | GND |  |
| RST | D22 |  |
| 3.3V | 3.3V |  |
| SSD1306 OLED (I2C) |  | ESP32 |  |
| VCC |  | 3.3V |  |
| GND |  | GND |  |
| SDA |  | D4 |  |
| SCL |  | D15 |  |

| Buzzer | ESP32 |
| --- | --- |
| + | D2 |
| - | GND |